# Example Configurations

These examples show typical Shelfbot configurations for different operation sizes, from small (500 SKUs) to high-volume (10,000+ SKUs). Examples include both single row and double row configurations.

## Small Operation (500 SKUs)

Profile:

- 500 active SKUs

- ~100 orders/day (~250 line items)

- Single-shift operation

Small configuration:

- Bins: ~975 bins (room for ~2 bins per SKU on average)

- Aisles/Robots: 2 robots in 2 aisles

- Racking: 2 aisles, 5 bays (13.5m), 13 levels (5.9m high)

- Footprint: 13.5m × 4.4m × 5.9m (L × W × H)

- Throughput capacity: 120 presentations/hour (2 robots × 60 presentations/hour per robot)

- Operators: 1 operator (works across 2 stations)

- Sufficient for: 100 orders/day (~30-40 presentations/hour with buffer)

View this Configuration →

## Medium Operation (2,500 SKUs)

Profile:

- 2,500 active SKUs

- ~400 orders/day (~1,000 line items)

- Single-shift operation

Medium configuration:

- Bins: ~2,600 bins (assumes ~1 bin per SKU on average)

- Aisles/Robots: 3 robots in 3 aisles

- Racking: 3 aisles, 10 bays (26.9m), 13 levels (5.9m high)

- Footprint: 26.9m × 6.2m × 5.9m (L × W × H)

- Throughput capacity: 180 presentations/hour (3 robots × 60 presentations/hour per robot)

- Operators: 1 operator (works across 3 stations)

- Sufficient for: 400 orders/day (~125-150 presentations/hour with buffer)

View this Configuration →

## Growth-Ready (5,000+ SKUs)

Profile:

- 5,000+ active SKUs

- ~800 orders/day (~2,000 line items)

- Single-shift operation with seasonal peaks

Double row configuration:

- Double rows: 10 rows (2 rows per robot — each robot serves both sides of its aisle)

- Bins: ~8,450 bins (high-density storage)

- Aisles/Robots: 5 robots in 5 aisles

- Racking: 5 aisles with 10 rows, 13 bays (35m), 13 levels (5.9m high)

- Footprint: 35m × 13.5m × 5.9m (L × W × H)

- Throughput capacity: 300 presentations/hour (5 robots × 60 presentations/hour per robot)

- Operators: 1 operator (works across 5 stations)

- Advantage: Higher storage density — each robot serves two dedicated rows instead of sharing a row

View this Configuration →

## High-Volume (10,000+ SKUs)

Profile:

- 10,000+ active SKUs

- ~1,600 orders/day (~4,000 line items)

- Multi-shift operation

High-volume configuration:

- Bins: ~10,000+ bins

- Aisles/Robots: 10 robots in 10 aisles (2 operators, 5 robots each)

- Racking: 10 aisles, 13 bays (35m), 13 levels (5.9m high)

- Footprint: 35m × 18.6m × 5.9m (L × W × H)

- Throughput capacity: 600 presentations/hour (10 robots × 60 presentations/hour per robot)

- Operators: 2 operators (each works across 5 stations)

- Supports: Peak throughput up to 1,600 orders/day

## Understanding the Numbers

### How bins are calculated

- Formula: Rows × Levels × Bays × 5 bins per bay

  - Single row mode: (N+1) rows where N = number of bots

  - Double row mode: 2N rows where N = number of bots (higher density)

- Bins per SKU: Most operations average ~1 bin per SKU, but this varies:

  - Fast-movers or high-stock items may need 2-3 bins

  - Slow-movers or small items may share a bin

- Safety stock: Add 10-20% extra bins for growth and flexibility

### How throughput scales

- Per robot: 60 presentations/hour

- Per operator: Up to 300 presentations/hour (with 5 robots)

- Multiple operators: Each operator works with their own cluster of 5 aisles

- Order line multiplier: Depending on your order profile, each bin presentation may fulfill 2-3 order lines — effectively doubling or tripling the pick rate relative to presentations

### Footprint breakdown

- Length (L): Number of bays × 2.69m per bay

- Width (W): Depends on row mode

  - Single row mode: (N+1) rows where N = number of bots

    - Formula: (N × 2.04m) + 0.84m

    - 2 bots = 4.9m, 3 bots = 7.0m, 5 bots = 11.0m, 10 bots = 21.2m

  - Double row mode: 2N rows where N = number of bots (each robot serves 2 dedicated rows)

    - Formula: 2.88m + (N-1) × 3.03m

    - 5 bots = 15.0m, 10 bots = 30.1m

- Height (H): Number of levels × 0.385m + base height

  - 13 levels = 5.9m (fits under 6m ceiling)

  - 16 levels = 7.3m (requires higher ceiling)

## Need Help Sizing Your System?

The fastest path is a data dive: send us your SKU master data + order history, and we'll provide a detailed sizing report within 5-10 business days.

What to send:

- SKU master (SKU, description, dimensions, weight, stock on hand)

- Order history (12 months preferred; minimum 30 days)

Read the Data Dive Specification

Or contact us directly:

- Email: sales@shelfbot.com

- Phone: +61 3 9069 5000

- Request site assessment

Back to Evaluator Pack| Technical Data Sheet | How it works