# System Overview

Shelfbot is a Goods-to-Person warehouse automation system that uses autonomous robots to retrieve storage bins from standard racking and deliver them to operators at stations. The system retrofits into existing AS4084-compliant steel storage racking and integrates with your WMS via REST API.

Key principle: We built Shelfbot to be simple to understand, simple to operate, and simple to maintain—without sacrificing performance.

## What Shelfbot Is

Shelfbot delivers the storage density and throughput of enterprise ASRS without the proprietary grids, 12-month implementations, or multi-million dollar commitments. It's designed for warehouses that need automation but can't afford the complexity or capital of traditional systems.

Core capabilities:

- Goods-to-Person picking (bins delivered to operators, not vice versa)

- 4× storage density improvement vs. manual operations

- 5× faster order picking vs. manual

- 4–8 week deployment (not 6–18 months)

- Robots as a Service (RaaS) model from $10,000/month

## System Components

### 1. Robots

- Autonomous navigation – Robots travel horizontally on rails and move vertically within racking aisles

- No batteries – Powered via 240V mains supply from outlets on the racking front frame (no downtime for charging)

- Payload – 20 kg max bin weight

- Speed – Up to 2,500 mm/s horizontal; vertical lift speeds optimized for safety and throughput

- Per-robot throughput – 60 bin presentations per hour

### 2. Bins

- Standard dimensions – 820 × 420 × 285 mm (L × W × H)

- Material – Plastic bins (standard); steel fire-rated bins available

- Items per bin – Depends on SKU dimensions; typically 5–50 items depending on product size

- Bins per level – 5 bins per shelf level (typical configuration)

### 3. Racking

- Standard steel racking – AS4084:2023 compliant storage racking (not proprietary grids)

- Aisle width – 1200 mm

- Height range – 4–10 m (up to ~21 levels at 10 m height)

- Aisle length – Up to 48 m (18 bays)

- Key advantage – Uses existing racking or standard racking suppliers; no proprietary infrastructure

### 4. Stations

- One per robot – At the end of each robot aisle, where the robot presents bins for picking and putaway

- Integrated weight verification – Platform scales confirm pick accuracy and catch overweight items

- Zones – A block of three or four robot aisles is served by one operator with one put-to-light trolley; the operator moves between the zone's stations as bins arrive

- Throughput per operator – Up to 300 bin presentations per hour with batch picking

### 5. Put-to-Light Trolley

- Batch picking – A 3 m put-to-light trolley behind the operator holds the zone's open orders, one shipper per position, so one presented bin serves several orders at once

- Put to light – The app shows the quantity to pick; one position lights at a time to show where to put it, eliminating picking errors

- Scan to bind – The runner scans each fresh shipper into a freed position; the system binds it to the next order and tracks completion

- Batch optimization – System optimizes order assignment and robot dispatch to keep all robots continuously working

- Completed order swap – When a position shows complete, the runner lifts the shipper onto the takeaway and places a fresh shipper of the size called; the operator keeps picking throughout

### 6. Server & Software

- On-premises server – Manages robot orchestration, bin locations, and WMS integration

- REST API – Secure API connects to your existing WMS, ERP, or e-commerce platform

- Real-time inventory sync – Product data, inventory levels, and order transactions synchronized via documented endpoints

- Cloud redundancy – System state backed up for disaster recovery

## How It Works

### 1. Inventory Storage

- Products are stored in bins on standard steel racking

- Each bin is assigned a unique ID and tracked by the system

- Multiple SKUs can share a bin, or SKUs can span multiple bins

- Bins are stored at high density across multiple aisle levels

### 2. Order Processing Flow

- Orders received – WMS sends orders to Shelfbot via API

- Batch optimization – System assigns orders to the open positions on each zone's trolley and optimizes robot dispatch

- Shipper induction – Runner places a fresh shipper of the size called in a freed position and scans it; the system binds it to the order

- Station direction – System directs operator to next ready station

- Source bin delivery – Robot delivers source bin containing required SKUs to station

- Sequential putting – Put-to-light lights one shipper position at a time with the quantity needed

- Pick confirmation – Operator scans the item and places it in the lit shipper position; weight verification confirms accuracy

- Source bin return – Operator initiates robot return when source bin complete

- Order completion – When the order is fully picked the position lights complete; the runner swaps the shipper onto the takeaway for packing and dispatch

- Inventory update – System updates inventory levels and notifies WMS

### 3. Putaway

- Manual putaway – Operator scans items into bins at the station

- Auto empty-bin flow – System detects empty bins and routes them for putaway

- FIFO support – System can prioritize older inventory for picking (first-in, first-out)

### 4. Scaling

- Add robots – Increase throughput by adding more robots to existing aisles (the put-to-light trolley keeps all robots continuously working)

- Add aisles – Expand storage capacity by adding more racking aisles

- Add operators – Each operator with a put-to-light trolley can handle up to 300 bin presentations per hour

- No system redesign required – Scale incrementally without rebuilding infrastructure

## Who These Docs Are For

### Operations Managers

- Evaluating whether Shelfbot fits your operation

- Understanding throughput, storage density, and ROI

- Planning deployment timelines and operational changes

### Engineers & Technical Teams

- Designing installations and integration points

- Reviewing electrical, racking, and environmental requirements

- Understanding API integration and data exchange

### Electricians & Fire Consultants

- Planning electrical supply and circuit protection

- Reviewing fire safety compliance and sprinkler requirements

- Understanding safety interlocks and guarding

### Procurement & Finance Teams

- Understanding capital vs. operational costs (RaaS model)

- Reviewing commercial terms and service agreements

- Planning budget allocation and payback timelines

## Documentation Structure

This page and its companions (Technical Data Sheet, Safety & Compliance, Integration, Example Configurations) are conceptual introductions. The engineering source of truth is the published documentation - the racking, electrical, data dive and other specifications - which is versioned and always current. Where a number here and a number in a published specification disagree, the specification wins.

## What's Not Covered Here

These docs are not operational manuals. For day-to-day operation, training, and troubleshooting:

- User Manual – Operator workflows, pick/putaway procedures, and troubleshooting

- Commissioning Manual – Installation and commissioning procedures (provided to installers)

- API Documentation – Endpoint specifications, authentication, and integration guide

## Next Steps

If you're evaluating Shelfbot:

- Read Technical Data Sheetto understand planning ranges

- Send us your SKU master + order history for a custom sizing (contact us)

- Book a site check to verify racking, electrical, and access requirements

If you're planning an installation:

- Review Racking Specificationand Electrical Specification

- Share site plans with us for validation

- Engage a fire consultant for sprinkler and compliance review

If you're integrating with your WMS:

- Review Integrationfor API requirements

- Confirm your WMS has documented APIs for products, inventory, and orders

- Schedule a technical integration workshop with our team

Specs & site requirements →| Get a site check | Integration overview →